Metrix MTX3250 Multimeter

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Metrix MTX3250 Multimeter.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Metrix MTX3250 Multimeter

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                    Function Name:

Initialize                        mtx3250_init

Configure

Voltage                        mtx3250_voltage

Current                        mtx3250_current

Frequency                      mtx3250_frequency

Resistance                     mtx3250_resistance

Capacitance                    mtx3250_capacitance

Temperature                    mtx3250_temperature

Mode

Rejection                      mtx3250_rejection

Filter                         mtx3250_filter

Input impedance                mtx3250_input_impedance

Ohm reference                  mtx3250_ref_ohm

Date / Time                    mtx3250_date_time

Beep / Contrast                mtx3250_beep_contrast

Print Mode                     mtx3250_set_print

Math Mode                      mtx3250_set_math

Store Mode                     mtx3250_set_store

Clear Store                    mtx3250_clear_store

Catalog Store                  mtx3250_catalog_store

Mem Store                      mtx3250_mem_store

Measurement                       mtx3250_measure

Error                             mtx3250_error

Close                             mtx3250_close

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

mtx3250_beep_contrast

int mtx3250_beep_contrast (int beep, double contrast);

Purpose

Change the Beep flag value and the contras of the instrument.

Parameter List

beep

Variable Type       int

Specifies the Beep of the instrument

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   1 - On

contrast

Variable Type       double

Specifies the Contrast of the instrument

Variable Type:  double

Variable Range:  0.0  -   0%

0.33 -  33%

0.67 -  67%

1.0  - 100%

Default Value:   1.0  - 100%

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_capacitance

int mtx3250_capacitance (int range);

Purpose

Switches the instrument to Capacitance measurement.

Parameter List

range

Variable Type       int

Specifies the range of the measurement.

Variable Type:  integer

Variable Range:  0 -  Auto

1 -  50 nF

2 -  500 nF

3 -  5 F

4 -  50 F

5 -  500 F

6 -  5000 F

7 -  50 mF

Default Value:   0 - Auto Range

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_catalog_store

int mtx3250_catalog_store (char *catalog);

Purpose

Returns the store-catalog of the instrument.

Parameter List

catalog

Variable Type       char (passed by reference)

Specifies the buffer to write the catalog.

Variable Type:  char *

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_clear_store

int mtx3250_clear_store (void);

Purpose

Clears the store of the instrument.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_close

int mtx3250_close (void);

Purpose

Closes the instrument.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_current

int mtx3250_current (int function, int range, int _2nd_display);

Purpose

Switches the instrument to current measurement.

Parameter List

function

Variable Type       int

Specifies the type of measurement.

Variable Type: integer

Valid Range:   0 -  DC

1 -  AC

2 -  AC+DC

Default Value: 0 -  DC

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range: -1 - Auto Peak

0 - Auto

1 - 500 A

2 - 5 mA

3 - 50 mA

4 - 500 mA

5 - 10 A

Default Value:   0 - Auto

_2nd_display

Variable Type       int

Specifies the second type of measurement.

Variable Type: integer

Valid Range:   0 -  Off

1 -  Period + Hz

4 -  Peak + C.F.

5 -  Peak Min + Peak Max

6 -  Ohm + Watt

Default Value: 0 -  Off

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_date_time

int mtx3250_date_time (char day, char month, char year, char hour,

char minute, char second);

Purpose

Change the date and time of the instrument.

Parameter List

day

Variable Type       char

Specifies the Day for the date of the instrument

Variable Type:  integer

Variable Range:  1 to 31

Default Value:   1

month

Variable Type       char

Specifies the Month for the date of the instrument

Variable Type:  integer

Variable Range:  1 to 12

Default Value:   1

year

Variable Type       char

Specifies the Year for the date of the instrument

Variable Type:  integer

Variable Range:  0 to 99

Default Value:   2

hour

Variable Type       char

Specifies the Hour for the time of the instrument

Variable Type:  integer

Variable Range:  0 to 23

Default Value:   0

minute

Variable Type       char

Specifies the Minute for the time of the instrument

Variable Type:  integer

Variable Range:  0 to 59

Default Value:   0

second

Variable Type       char

Specifies the Second for the time of the instrument

Variable Type:  integer

Variable Range:  0 to 59

Default Value:   0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_error

int mtx3250_error (char errorMessage[]);

Purpose

Returns the code error

Parameter List

errorMessage

Variable Type       char []

Specifies the buffer to write the error message.

Variable Type:  char *

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_filter

int mtx3250_filter (int filter);

Purpose

Changes the Filter of the instrument.

Parameter List

filter

Variable Type       int

Specifies the rejection of the instrument

Variable Type:  integer

Variable Range:  1 - On

0 - Off

Default Value:   0 - Off

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_frequency

int mtx3250_frequency (int range, int voltageRange, int _2nd_display);

Purpose

Sets the instrument to frequency measurement.

Parameter List

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  0 - Auto

1 - 5 Hz

2 - 50 Hz

3 - 500 Hz

4 - 5 kHz

5 - 50 kHz

6 - 500 kHz

7 - 5 MHz

Default Value:   0 - Auto

voltageRange

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  1 - 500 mV

2 - 5 V

3 - 50 V

4 - 500 V

5 - 600 V

Default Value:   2 - 5 V

_2nd_display

Variable Type       int

Specifies the second type of measurement.

Variable Type: integer

Valid Range:   0 -  Off

7 -  Period + Duty+

8 -  Period + Duty-

Default Value: 0 -  Off

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_init

int mtx3250_init (int serial_port, int baud, int IDQuery,

int resetDevice);

Purpose

Panel Name:    Initialize

Description:   Initializes the instrument in the

following ways:

- opens the instrument

- optionally performs an

identification query on the

instrument

- optionally sets the instrument

to a default configuration

Parameter List

serial_port

Variable Type       int

Control Name:  Serial Port

Description:   Specifies the serial port to be used to

communicate with the instrument.

Variable Type: Integer

Valid Range:   1 to 4

Default Value: 1

baud

Variable Type       int

Control Name:  Baud

Description:   Specifies the baud of serial port to

communicate with the instrument.

Variable Type: Integer

IDQuery

Variable Type       int

Control Name:  ID Query

Description:   Specifies if an ID Query is sent

to the instrument during the

initialization procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No

Default Value: 1 - Yes

resetDevice

Variable Type       int

Control Name:  Reset

Description:   Specifies if the instrument is to

be reset during the initialization

procedure.

Variable Type: Integer

Valid Range:   0 = No Reset

1 = Reset device

Default Value: 1 = Reset

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_input_impedance

int mtx3250_input_impedance (int input_impedance);

Purpose

Changes the Input impedance of the instrument. It

is used for measurement in the mV range.

Parameter List

input_impedance

Variable Type       int

Specifies the input impedance for measurement in

the mV range

Variable Type:  integer

Variable Range:  0 - 10 MOhm

1 - 1 GOhm

Default Value:   0 - 10 MOhm

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_measure

int mtx3250_measure (double *measure1, char type1[], double *measure2,

char type2[], double *measure3, char type3[]);

Purpose

Returns the measurements of the instrument.

Parameter List

measure1

Variable Type       double (passed by reference)

Displays the results of the measurement in

Scientific Format.

Variable Type:  double

type1

Variable Type       char []

Specifies the buffer to write the unit of the measurement 1.

Variable Type:  char *

measure2

Variable Type       double (passed by reference)

Displays the results of the measurement in

Scientific Format.

Variable Type:  double

type2

Variable Type       char []

Specifies the buffer to write the unit of the measurement 2.

Variable Type:  char *

measure3

Variable Type       double (passed by reference)

Displays the results of the measurement in

Scientific Format.

Variable Type:  double

type3

Variable Type       char []

Specifies the buffer to write the unit of the measurement 3.

Variable Type:  char *

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_mem_store

int mtx3250_mem_store (int mem, char value[], int number_ofBytes);

Purpose

Returns a Store-memory of the instrument.

Parameter List

mem

Variable Type       int

Specifies the memory of Store Menu

Variable Type:  integer

Variable Range:  0 to 9

Default Value:   0

value

Variable Type       char []

Specifies the buffer to write the memory.

Variable Type:  char *

number_ofBytes

Variable Type       int

Specifies the maximum number of bytes to read a memory.

One point uses 40 bytes (for one measure) or 65 bytes (for three

measures)

Variable Type:  integer

Default Value:   500

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_ref_ohm

int mtx3250_ref_ohm (int reference, int value);

Purpose

Change the OHM value reference of the instrument.

It is used for dBm or resistive power measurement.

Parameter List

reference

Variable Type       int

Specifies the Reference to change

Variable Type:  integer

Variable Range:  0 - W

1 - dBm

Default Value:   1 - dBm

value

Variable Type       int

Specifies the value of the reference

Variable Type:  integer

Variable Range:  0 to 10000

Default Value:   1

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_rejection

int mtx3250_rejection (int rejection);

Purpose

Changes the Rejection of the instrument (50 or 60

Hz).

Parameter List

rejection

Variable Type       int

Specifies the rejection of the instrument

Variable Type:  integer

Variable Range:  0 - 50 Hz

1 - 60 Hz

Default Value:   0 - 50 Hz

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_resistance

int mtx3250_resistance (int measure, int range);

Purpose

Sets the instrument to Resistance measurement.

Parameter List

measure

Variable Type       int

Specifies the measurement

Variable Type:  integer

Variable Range:  0 -  Resistance

1 -  Continuity testing

2 -  Diode testing

Default Value:   0 -  Resistance

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  0 -  Auto

1 -  500  Ohm

2 -  5 kOhm

3 -  50 kOhm

4 -  500 kOhm

5 -  5 MOhm

6 -  50 MOhm

Default Value:   0 - Auto

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_set_math

int mtx3250_set_math (int mode, int function, int range,

double AParameter, double BParameter, double coef,

char unit[]);

Purpose

Changes the Math-mode values of the instrument.

Parameter List

mode

Variable Type       int

Specifies the mode of Math Menu

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

function

Variable Type       int

Specifies the function of Math Menu

Variable Type:  integer

Variable Range:  0 - Voltage

1 - Current

2 - Resistance

3 - Frequency

Default Value:   0 - Voltage

range

Variable Type       int

Specifies the range of function of Math Menu

Variable Type:  integer

Variable Range:  0 - 500mV / 500A / 500Ohm / 5Hz

1 - 5V / 5mA / 5kOhm / 50Hz

2 - 50V / 50mA / 50kOhm / 500Hz

3 - 500V / 500mA / 500kOhm / 5kHz

4 - 600V / 10A / 5MOhm / 50kHz

5 - 50MOhm / 500kHz

6 - 5MHz

Default Value:   0 - 500mV / 500A / 500Ohm / 5Hz

AParameter

Variable Type       double

Specifies the A-parameter of Math Menu

Variable Type:  double

Variable Range:  -9.999e+9 to 9.999e+9

Default Value:   0.0

BParameter

Variable Type       double

Specifies the B-parameter of Math Menu

Variable Type:  double

Variable Range:  -9.999e+9 to 9.999e+9

Default Value:   0.0

coef

Variable Type       double

Specifies the coefficient of Math Menu

Variable Type:  double

Variable Range:  1e-9 - 'n'

1e-6 - ''

1e-3 - 'm'

1.0  - ''

1e3  - 'k'

1e6  - 'M'

1e9  - 'G'

Default Value:   1.0 - ''

unit

Variable Type       char []

Specifies the unit of Math Menu

Variable Type:  char *

Variable Range:  "A" to "Z", "Ohm", "Hz"

Default Value:   "A"

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_set_print

int mtx3250_set_print (int mode, double timer);

Purpose

Changes the Timer value of the instrument. It is

used for repetitive measurement.

Parameter List

mode

Variable Type       int

Specifies the mode of Print Menu

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

timer

Variable Type       double

Specifies the value of timer

Variable Type:  double

Variable Range:  0.0 to 36000 (10:00:00)

Default Value:   1.0

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_set_store

int mtx3250_set_store (int mode, double timer, int depth);

Purpose

Changes the Store-mode values of the instrument.

Parameter List

mode

Variable Type       int

Specifies the mode of Store Menu

Variable Type:  integer

Variable Range:  0 - Off

1 - On

Default Value:   0 - Off

timer

Variable Type       double

Specifies the value of timer

Variable Type:  double

Variable Range:  0.0 to 36000 (10:00:00)

Default Value:   1.0

depth

Variable Type       int

Specifies the depth of Store Menu

Variable Type:  integer

Variable Range:  1 to 2021

Default Value:   1

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_temperature

int mtx3250_temperature (int unit, int sensor);

Purpose

Sets the instrument to Temperature measurement.

Parameter List

unit

Variable Type       int

Specifies the unit of temperature.

Variable Type:  integer

Variable Range:  0 - C

1 - F

2 - K

Default Value:   0 - C

sensor

Variable Type       int

Specifies the sensor selection.

Variable Type:  integer

Variable Range:  0 - Pt 1000

1 - Pt  100

Default Value:   1 - Pt  100

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

________________________________________________________________________________

mtx3250_voltage

int mtx3250_voltage (int function, int range, int _2nd_display);

Purpose

Switches the instrument to voltage measurement.

Parameter List

function

Variable Type       int

Specifies the type of measurement.

Variable Type: integer

Valid Range:   0 -  DC

1 -  AC

2 -  AC+DC

Default Value: 0 -  DC

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range: -1 - Auto Peak

0 - Auto

1 - 500 mV

2 - 5 V

3 - 50 V

4 - 500 V

5 - 600 V

Default Value:   0 - Auto

_2nd_display

Variable Type       int

Specifies the second type of measurement.

Variable Type: integer

Valid Range:   0 -  Off

1 -  Period + Hz

2 -  dB + Hz

3 -  ref.Ohm + dBm

4 -  Peak + C.F.

5 -  Peak Min + Peak Max

6 -  Ohm + Watt

Default Value: 0 -  Off

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out